因产品不断更新，设计规格如有变更，恕不另行通知；

表列有关数据仅供参考，具体数据以实际产品配备为准。

BECAUSE OUR PRODUCT INNOVATION,THE MODE

SPECIFICATION WILL CHANGE,ALL PRODUCT MODE INDEX ACCORDING ACTUAL DATE.

Product Catalogue

www.jzsk.com.cn

# HANYANG

CNC MACHINE TOOL 汉洋数控

智能◎高速◎高精◎ 高光

泰州市汉洋数控机床制造有限公司

TAIZHOU HANYANG CNC MACHINE TOOL MANUFACTURE CO.,LTD.

地址：江苏省泰州市九龙镇龙翔路168号 ADD:No.168 longxiang Road,Jiulong Town,Taizhou City,Jiangsu

电话：0523-8656279986551641      TEL:0523-8656279986551641

传真：0523-82212328               FAX:0523-82212328

网 址 ：www.jzsk.com.cn                      Http://en.jzsk.com

邮箱：sales@jzsk.com.cn                         E-mail:sales@jzsk.com.cn

邮编：225300                       P.C:225300

…  … …  … …   …

-45T

HYSK

◎ 瑞 禾 广 告  1  3  9  5  2 6  1  7  1  1  0

汉洋数控

企业简介

CORPORATE BRIEF

专业铸就品牌品质成就未来 Precise Machining Makes CNC Future

汉洋数控是国内专业生产数控电火花线切割机床、数控 加工中心、数控铣床、数控雕铣机、数控砂线切割机床 等数控设备的高新技术企业，公司具有先进的产品研发 设计和生产制造能力。

主要产品有数控砂线切割机床、 DK77 系列数控电火花 线切割机床、D71 系列电火花成形机、电火花小孔加 工机、数控雕铣机，同时应客户要求生产各种专用设

备 。

高素质的员工队伍，科学的管理，与大中院校科研机构 的联合，使我公司的各种机床性能稳定可靠，能满足各 种模具及机械零件的加工制造。

本公司通过了ISO9001:2008 国际质量体系认证，产品 已通过CE 认证，销往国内二十多个省市，并出口马来 西亚、韩国、日本、泰国、黎巴嫩、印度、智利、伊 朗、埃及、巴基斯坦、新加坡、乌克兰、俄罗斯等三十 多个国家。产品实行“三包”,终身维修，免费为用户 提供技术培训，售后服务及时周到，在全国各地设有几 十家售后维修办事处，在韩国、越南、泰国、日本、俄 罗斯设有销售及售后服务办事处。

HANYANG

CNC MACHINE TOOL 汉洋数控

Hanyang CNC manufacturing company is one of the biggest factory  in China with strong  R&D capability,specially  manu- facture CNC EDM wire cutting machine tools,CNC machining centers,CNC  milling  machines,CNC  engraving  and  milling machines,CNC  sand  wire  cutting  machine  tools  and  other CNC equipment.The company has advanced product devel-  opment,design and manufacturing capabilities.

The main products are CNC sand wire cutting machine tools, DK77 series CNC wire cutting machine tools,D71 series EDM forming machines,EDM small hole processing machines,CNC engraving  and  milling  machines,and  various  special  equip- ments are produced according to customer requirements.

The   high-quality  workforce,scientific   management,and   the combination  of scientific  research  institutions  in  universities and  colleges   make  our  company's  various   machine  tools stable and reliable,which can meet the processing and manu- facturing of various molds and mechanical parts.

The   company   has   acquired   CE   certificate,ISO9001:2008 international quality system certification,and its products have passed CE certification.They are sold to more than 20 domes- tic   provinces   and   cities,and   exported   to   Malaysia,South Korea,Japan,Thailand,Lebanon,India,Chile,Iran,Egypt,

Pakistan,More than 30 countries including Singapore,Ukraine and   Russia.The   product   implements   "three   guarantees', lifelong  maintenance,free  technical  training  for  users,timely and   thoughtful   after-sales   service,dozens   of   after-sales maintenance offices across the country,sales and after-sales in Korea,Vietnam,Thailand,Japan,and Russia Service office.

资质证书

QUALIFICATION CERTIFICATE

实用新型专利证

实用新型专

9Arausnrnr                                              实用新型十

里 A       mnr季

实用新型专利：

a     r

4

Ar

nr           后

乐

实用新型专利证书

u

r      回

少

专相相     isi    附

年

解     放

长

长

般

中 南

| 实用新型专利证牛 态 | 号 中 | A9n 星 8 m 区 实用新型 1 实 用 新 型 意 LTa 季 ra: 盒 3 n. 的角 树 usn 长 关 s 警 海 临 AT. 解 外观设计专 专 解 播 : 9 t 专利 证 书 华 多 r 青 8 |
| --- | --- | --- |

OVC

EUMOPEAJN COMMLMTY-CEKTOCATEOFADEQLACY

canrcen                         saancBE/oIe1

三 二 一

-

三

C∈=R    i    O

CERTFICATE

企业

新技

证 书

诺 号

aana4

电加工机床达标认证证书

:*nn

r

eumes***

守合同重信用企业

公 示 证 书

9E

aVC

cnIcn             usac             aensG/ne₁

三

三 三

二二

三

(∈:

质量管理体系认证证书

*

堡三

中国质盟认证中心

泰州市知名商标证书

编号：

翠州非汉浮数拦机床制造有固会司：

你 单 位 使 用 在      B87        商品(服务) 上的#     商标《注册证号 mm      )经我局 认定为秦州市知名商标，有效期两年。

中平 和

二二

授予：秦州市汉律数控模床床制造有阻会司

荣誉证书

江苏省质量管理先进企业

江苏省民营科技企业

经母认释合《C 器省发届民营料接企业角例》的有关概定。 情发此证。

法定代表人：

企 业 地 址 ： *  A 电  光

江苏省

6*

aVC

EUHOPLAS CDMMUNITY-CEKTFCATE OFADEOUACY

amMWCNTE5wK8/LDK7SE/Lm!

三      三

二  二

CE

P

,  D    地

脚

职业健康安全管理体系认证证书

年制造布限会司

泰州名牌产品证书

TAIZHOU  FAMOLSBRANDCERTIPCATE

泰州市汉详数拉机床制造有限公司

你单位生产的下列产品被泰州市

名牌战略推进委员会认定为2006年 泰州名牌产品，特发此证。

泰州市质量管理奖证书

TAIZHOUQUALTY MANAGMENTHONOUR

企业名称：本兴市以斗我牧轨床制进有母

经泰州市质量奖审定委员会审 定 ， 你 单 位 荣 获 二 0 0 六 年 泰 州 市  质 量 管 理 奖 称 号 ( 有 效 期 三 年 ) ,   特 发 此 证 。

三

质量管理体系认证证书

环境管理体系认证证书

州市汉洋数控机库制造有限公司

标准化良好行为证书

烈阻斗数放机东珠有服会司：

u.n0,,

08/T  4-200重，边 hAA 磁书有败附里 0  年 " 月 "B

an

面家标准化督理墨员会制

●●●●●●

六轴伺服中走丝线切割机床

6-Axis EDM Wire Cutting Machine DK77HZ Series

| 超越中走丝，替代慢走丝! INSTEAD OF SLOW SPEED WIRE CUTTING, BETTER THAN MIDDLE SPEED WIRE CUTTING ◎嵌入式三色灯，节省空间。 Embedded three-color light,saving space. ◎工作区域采用LED照明，方便操作。 The working area adopts LED lighting for easy operation. | high-strength taper processing. 锥度结构 Taper structure ◎慢走丝结构，确保高强度锥度 加工的高精度。 machine ensures high precision of The structure of slow speed wire cutting ◎Z轴采用高精度双直线导轨， 长期加工稳定性、精准性更高。 lTihneearZ-axguisidusese,swhihcih- pprerocvisidioen dhoiugbheler long-term processing stability and accuracy. |
| --- | --- |

………

……

◎操作带进口电子手脉，可手动调 整加工距离，使用方便。

…  …

The operation is equipped with an import- ed electronic hand pulse,which can manu- ally adjust the machining distance for easy operation.

◎油槽升降，节约空间，方便美观。 The oil tank can be raised and

lowered to save space,and is convenient  and  beautiful.

SF-45T

e⁷o⁷0

HYSK

汉洋数控

◎机床主体部件轴承均采

用日本进口NSK精密级轴承

The main bearings of the machine

◎机体结构采用慢走丝的T 型主体结构，刚性高，承 载能力强

The machine body adopts T-shape

01

and strong load-bearing capacity.

main structure with strong rigidity

tool are allJapan NSK precision bearings.

◎UVZ 轴伺服电机

UVZ axis adopts servo motor.

自动涨丝机构

Automatic wire tension mechanism

◎数字调节不同张力的自动紧 丝装置，对不同丝径快速调 节，方便快捷。

The     automatic    wire    tightening device  with   digital   adjustment   of

ddfferentfferent tenwisrnicaamnetqeursic,klwiadchjuisst

convenient and fast.

◎配置进口高精度滚珠丝

杠，精度高，使用寿命长。

cision   ball   screw,high   precision

and long service life.

Equipped with  imported  high-pre-

HANYANG

CNC   MACHINE   TOOL 汉洋数控

| while power off 自动紧丝 Automatic tightening ◎非电式自动紧丝装置，使 用方便、可靠。外网断电， 运丝瞬间停止，保护丝筒钼 丝不会过冲。 Non-electric automatic tightening wire running stopped immediately device,easy to use,reliable,and | etc.)to ensure the service life of the moving parts,and waste oil can be automatically recycled. cutting. 集中润滑 Centralized lubrication ◎自动润滑系统，对机床所有运动 部件(导轨、丝杆、丝筒等)定时润 滑，确保运动部件的使用寿命，同 时废油可自动回收。 lubricates all moving parts of the machine tool(guide rails,screw rods,wire barrels, The automatic lubrication system regularly 伺服电机 Servo Motor ◎机床采用进口伺服电机 传动，6轴数控，5轴联动 切割 ed servo motor transmission, 6-axis CNC,and 5-axis linkage The machine tooladopts import- |
| --- | --- |

| ◎配置进口高精度直线导 轨 ， 精度高 ， 使用寿命长 。 Equipped with imported high-lin- ear slider,high precision and long service life. | 防锈工作台 Rust-proof Workbench ◎防高蚀、防生锈钢件工作 台，确保工作台长期使用的 高 精 度 。 The anti-corrosion and anti-rust steel workbench ensures the high precision of the workbench during long-term use. |
| --- | --- |

高精度软件 HIGH PRECISION CONTROLLING

基于慢走丝切割运动算法的控制软件

Software  based  on  slow  wire  cutting   motion  algorithm

| 8837675Y⁰800U⁰0y o027.02 611×03863 Voze o062:×0026625 Y0iao eae 文如E et 友好的人机界面，加工图形可旋转 或镜像等操作，系统支持X、Y、U 、V 、Z 轴伺服及运丝电机伺服。 The friendly human-machine interface,with the ability to do rotate or mirror processing The system supports X,Y,U,V,Z-axis with servo and wire feeding with servo motor. | 支持DXF 文件的读取，可与AUTO- CAD、CAXA等软件无缝对接。 优化插补及伺服跟踪处理算法，有 效提高工件轮廓上精度。 Support reading DXF files and integrating with software such as AutoCAD and CAXA. Optimize interpolation and servo tracking the accuracy of workpiece contours. processing algorithms to effectively improve | 简洁明了的IO 监控功能，方便实时了 解机床的相关工作情况。 The concise I0 monitoring function facilitates real-time reading of the relevant working conditions of the machine tool. |
| --- | --- | --- |

| 方便直观的轨迹生成方式，屏幕实 时给出对应的操作提示。 Convenient and intuitive trajectory genera- tion method,with real-time operation prompts provided on the screen. | 强大的厂商工艺库和用户工艺库， 方便随时调取及修改。 Powerful manufacturer process database and user process database,easy to access and modify at any time. | 支持5种不同的语言。 Supporting 5 kinds of language |
| --- | --- | --- |

实用的锥度相关参数计算功能。

Practical taper related parameter calculation

function.

CNC MACHINE TOOL 汉洋数控

主要技术参数    MAIN TECHNICAL DATA

| 型号 Type 单位 Unit DK7735HZ DK7745HZ DK7755HZ |  |
| --- | --- |
| 工作台尺寸Worktable size (mm) 470×680 570×780 650×1060 |  |
| 工作台XY 行程X,Y axis travel(X*Y) (mm) 350×450 450×550 550×850 |  |
| 最大加工厚度Z Max.cut thickness Z (mm) 300 300 400 |  |
| U,V 轴行程U,V axis travel (mm) 60×60 |  |
| 电极丝直径 Diameter of Mo.wire (mm) 钼丝(Molybdenum wire)φ0.12~φ0.18 |  |
| 走丝速度Wire speed 7档可调 |  |
| 加工斜度/工件厚度 Taper angle/workpiece thickness (°/mm) 3°/60mm |  |
| 加工精度 Process accuracy(vertical) (mm) | 多次切割(Multi-cut):10×10×30 四 方(Squire)≤0.006 一次切割(One-cut):≤0.012 八方(Octagon)≤0.009 |
| 加工面粗糙度 Process roughness (μm) 多次切割(Multi-cut):Ra≤0.8 一次切割(One-cut):Ra≤2.5 |  |
| 驱动电机 Motor drive system 伺服马达Opition:Servo motor |  |
| 丝杠与导轨 Screw &Guides(X,Y) 高精密滚珠丝杠/直线滑动导轨 Precision ball screw/Linear motion guides |  |
| 张力系统Wire tension 重锤自动张紧机构 |  |
| 工作液/液箱容量Working fluid/capacity (L) 复合型或水溶性专用液Complex or water-dissovle special coolant/65L |  |
| 供电电源Power supply AC3-φ380V 2KW |  |
| 最大工件重量Max.table carry (kg) 600 800 1000 |  |
| 主机净重Net weight (kg) 1800 2200 3200 |  |
| 外形尺寸Dimensions (mm) 1800×1260×2100 2022×1460×2100 2222×2160×2300 |  |

豪华智能型中走丝线切割机床

DK77HC Series Medium Speed wire Wedm

| 往复走丝型 ◎运丝采用变频器实现走丝速度无极调整，实现低噪音平稳运 行； ◎运丝采用PLC 控制实现变形程往复走丝，正反向走丝行程任 意设定； ◎实现无条纹切割，提高加工工件表面质量。 Reciprocating wire type ◎The use of frequency converter to achieve wire speed stepless adjustment,to achieve smooth operation with low noice; ◎Run wire using PLC control to achieve variable stroke reciprocating wire,forward and reverse travel for any setting; ◎Realize stripe-free cutting,improve the surface machining quality of workpiece.. | 第五轴功能 Funtion of Fifth Axis ◎在X、Y、U、V 轴基础上增加第五轴。实现工件旋转切割加工； ◎标配电子手轮控制，方便用户调整和操作。 ◎Add the fifth axis on the basis of X,Y,U,Vaxis.To achieve the workpiece rotation cutting processing; ◎Standard electronic hand wheel control,user-friendly adjustment and operation. |
| --- | --- |

HANYANG

CNC MACHINE TOOL 汉洋数控

主要技术参数    MAIN TECHNICAL DATA

型号Type

工作台尺寸Worktable size

工作台XY 行程X,Y axis travel(X*Y)   最大加工厚度Z Max.cut thickness Z U,V轴行程 U,V axis travel

电极丝直径 Diameter of Mo.wire

走丝速度Wire  speed

加工斜度/工件厚度 Taper angle/workpiece

加工精度 Process accuracy(vertical)

加工面粗糙度 Process roughness

驱动电机 Motor drive system

丝杠与导轨 Screw  &Guides(X,Y)

单位 Unit

(mm)

(mm)

(mm)

(mm)

(mm)

DK7732HC                          DK7740HC                          DK7750HC

370×610                                 480×720                                   580×880

300×400                                 400×500                                   500×600

300                                           300                                            350

60×60

钼丝(Molybdenum    wire)φ0.12~φ0.18

7档可调

thickness (°/mm)

(mm)

(μm)

3°/60mm

多次切割(Multi-cut):10×10×30      四方(Squire)≤0.006

一次切割(One-cut):≤0.012 八方(Octagon)≤0.009

多次切割(Multi-cut):Ra≤1.2 一次切割(One-cut):Ra≤2.5

步进电机 Stepper motor(选配：伺服马达 Opition:Servo motor)

高精密滚珠丝杠/直线滑动导轨 Precision  ball  screw/Linear  motion  guides

张力系统Wire  tension                                                                                                               精密弹簧自动张紧机构

工作液/液箱容量Working fluid/capacity           (L)    复合型或水溶性专用液Complex or water-dissovle special coolant/65L

| 供电电源Power supply |  |  | AC 3-φ380V 2KW |  |
| --- | --- | --- | --- | --- |
| 最大工件重量Max.table carry | (kg) | 300 | 400 | 500 |
| 主机净重Net weight | (kg) | 1900 | 2200 | 2500 |

| 外形尺寸Dimensions | (mm) | 1800×1150×2120 | 1950×1350×2150 | 2100×1500×2300 |
| --- | --- | --- | --- | --- |

HANYANG

CNC MACHINE TOOL

汉洋数控

DK77 系列数控电火花线切割机床

DK77 Series CNC wire Wedm

DK7780

DK7740 (环保型)

DK77120(分体式)

07                                                                                                                                                             08

电火花线切割机床数控柜台

NC Control UNIT OF DK77 Series CNC Wire EDM

■

我公司DK77 系列数控电火花控制柜有单板机和编控一体两大类， 其中编控一体有HF、  HL、PM-K 等编控系统供选择，编程操作方 便，功能强大，同时有台式和立柜两种供选择。

We have 2 categories of CNC EDM control carbinet for DK77 series: single  board  type  and  integration  type  of  programming  &  controlling,of  which   there   are   different   programmable   and  controllable system for choice,such as HF、HL、and PM-K etc. Convenient   programming   and   operation   leads   to   powerful function.And there are desk cabinet type and vertical cabinet for choice.

09

HANYANG

CNC MACHINE TOOL 汉洋数控

主要技术参数    MAIN TECHNICAL DATA

| 型号 | 工 作 台 面 尺 寸 | 工作台行程 | 最 大 加 工 厚 度 加工锥度(选配) |  | 最大工件重量 | 主 机 净 重 | 外 形 尺 寸 | 供电电源 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Worktable size (mm) | Worktable travel (mm) | Max.cut thickness (mm) | Taperloptional) | Max.table carry (kg) | Net weight (kg) | Dimensions (mm) | Power supply |
| DK7720 | 270×420 | 200×250 | 200 | 6°/80mm | 100 | 800 | 1160×880×1400 |  |
| DK7725 | 340×520 | 250×220 | 400 | 6°~12°/80mm | 200 | 1200 | 1485×1010×1700 |  |
| DK7730 | 340×560 | 300×360 | 400 | 6°~12°/80mm | 200 | 1250 | 1485×1050×1700 |  |
| DK7732 | 380×600 | 320×400 | 400 | 6°~30°/80mm | 300 | 1400 | 1640×1280×1700 |  |
| DK7735 | 380×650 | 350×450 | 400 | 6°~30°/80mm | 300 | 1440 | 1660×1330×1700 |  |
| DK7740 | 410×710 | 400×500 | 400 | 6°~60°/80mm | 450 | 1600 | 1830×1490×1700 |  |
| DK7740B | 450×840 | 400×630 | 400 | 6°~60°/80mm | 450 | 1670 | 1845×1610×1700 |  |
| DK7745 | 500×750 | 450×550 | 400 | 6°~60°/80mm | 450 | 1650 | 1865×1520×1700 | AC380V/220V |
| DK7755 | 596×880 | 550×650 | 500 | 6°~60°/80mm | 800 | 2500 | 2070×1170×1790 | 50Hz |
| DK7755B | 596×1060 | 550×850 | 500 | 6°~60°/80mm | 850 | 2600 | 2070×1950×1790 | ≤2KW |
| DK7763 | 790×1260 | 630×1000 | 600 | 6°~60°/80mm | 1000 | 3500 | 2265×2160×1930 |  |
| DK7763A | 720×1260 | 630×1000 | 600 | 6°~60°/80mm | 1000 | 3200 | 2200×2160×1930 |  |
| DK7780 | 900×1500 | 800×1200 | 600 | 6°~60°/80mm | 1500 | 5500 | 2900×2500×2150 |  |
| DK7780A | 880×1440 | 800×1200 | 600 | 6°~60°/80mm | 1500 | 4700 | 2700×2300×1900 |  |
| DK7780B | 880×1260 | 800×1000 | 600 | 6°~60°/80mm | 1500 | 4500 | 2620×2260×1900 |  |
| DK77100 | 1010×1500 | 1000×1200 | 600 | 6°~12°/80mm | 1500 | 5600 | 2930×2500×2000 |  |
| DK77120 | 1300×1900 | 1200×1600 | 600 | 6°~12°/80mm | 4000 | 10000 | 4200×3700×2000 |  |
| DK77160 | 1400×2500 | 1600×2000 | 800 | 6°~12°/80mm | 5000 | 12000 | 5000×4200×2200 |  |

10

电火花成型机床

D71 Series DIE SINKING Wedm

★高钢性机械结构

★简便的操作界面，易于掌握

★专家系统，能使电损耗降低

★超细的加工表面，并配有镜面加工回路

★高灵敏度的防积碳电路

★模块化电器设计

★High rigidity mechanical structure

★ Easy to operate interface,easy to operate

★ Expert datebase system reduce power loss

★ Ultra fine surface processing,equipped with

mirror processing circuit

★ High senstivity anti carbon deposition circuit

★ Modular electrical design

D7145

该系列机床与脉冲电源柜配套使用，Z 轴配日本三洋伺服电机，具有Z 轴数控的特点。 三轴联动电火花成形机X 、Y轴坐标采用精密滚珠丝杠，精密光栅数显进行坐标显示， 坐标定位精度高。三轴联动电火花成形机电柜采用先进的工业工控主板，功能强、稳 定性好，操作简单，只要输入Z 轴深度等少数参数，即能从丰富的专家库系统中调出 相匹配的加工数据，由粗加工到精加工一次自动完成。

This series of machine tools is used in conjunction with a pulse power cabinet.The Z axis is equipped with a Japanese Sanyo servo motor,which has the characteristics of Z axis numerical  control.The  Xand Y  axis  coordinates  of  the  three-axis  linkage  EDM  machine adopts  precise  ball screw,precise grating  DRO for coordinate display,and the coordinate positioning   accuracy   is   high.The   three-axis   linkage   EDM   electromechanical   cabinet adopts   advanced   industrial   industrial   control   mainboard,which   has   strong   functions, good stability  and simple operation.As  long  as  a  few  parameters  such  as  Z-axis  depth are  input,the  matching  processing  data  can   be   retrieved  from  the   rich  expert   library system.It could  be  automatically  completed  in  one  time for  roughing to finishing.

HANYANG

CNC MACHINE TOOL 汉洋数控

MAIN TECHNICAL DATA

主要技术参数

| 型号Type | 单位Unit | D7125 | D7132 | D7140 | D7145 | D7150 | D7160 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 工作台尺寸(长×宽) Worktable size(L×H) | (mm) | 400×250 | 500×320 | 650×400 | 720×450 | 800×500 | 1000×600 |
| 工作槽(长×宽×高) Worktable grooves(L×W×H) | (mm) | 1030×560×320 | 1030×560×320 | 1350×630×410 | 1400×710×410 | 450×780×500 | 1680×850×580 |
| 左右行程-X轴 Xaxis travel | (mm) | 250 | 320 | 400 | 450 | 500 | 600 |
| 前后行程-Y轴 Yaxis travel | (mm) | 150 | 250 | 300 | 350 | 400 | 450 |
| Z轴行程 Xaxis travel | (mm) | 240 | 240+240 | 250+250 | 250+250 | 250+300 | 250+350 |
| 电 极 头 平 面 与 工 作 台 最 大 距 离 Distance between letrode &worktable | (mm) | 400 | 550 | 650 | 680 | 700 | 800 |
| 最大工件重量 Max.workpiece weight | (kg) | 250 | 550 | 750 | 800 | 800 | 1000 |
| 最大电极重量 Max.electrode weight | (kg) | 50 | 60 | 70 | 80 | 100 | 150 |
| 机床重量 Machine weight | (kg) | 1100 | 1200 | 1800 | 2000 | 2200 | 3000 |
| 机床尺寸 Machine dimension | (mm) 1100×1200×2040 100×1400×2120 1500×1500×2250500×1500×2250 600×1500×2300 1700×1600×2400 |  |  |  |  |  |  |

| 电箱Type | 单位Unit | 60 ZNC | 75 ZNC |
| --- | --- | --- | --- |
| 最大加工电流 Max.maching electric current | (A) | 60 | 75 |
| 最大加工速度 Max.machining speed | (mm³/min) | 400 | 600 |
| 最小电极消耗比 Min.electrode wear |  | 0.1% | 0.1% |
| 最佳表面粗糙度 Best surface roughness | (μm) | Ra0.8 | Ra0.8 |
| 电源消耗 Power consumption | (KW) | 6 | 9 |
| 重量 Weight | (kg) | 200 | 250 |
| 尺寸(长×宽×高) Dimension(L×W×H) | (mm) | 600×650×1800 | 600×650×1800 |

11                                                                                                                                              12

HANYANG

三轴数控电火花穿孔机

DD703 Series 3-axis CNC spark-edm

三轴数控穿孔机的适用范围 ：

主要技术参数    MAIN TECHNICAL DATA

CNC MACHINE TOOL 汉洋数控

采用电极管(黄铜管，紫铜管)作为工具电极利用电火花放电 蚀除原理，在电极(空心铜管)与工作之间施加高频脉冲电源 形成小脉宽，大峰值电流的放电加工，辅以汽动高压泵冷却 排渣。使工件的蚀除速度大大加快，特别适用在不锈钢、淬 火钢、铜、铝、硬质铝合金等各种导电材料上的加工直径 0.15mm-3.0mm 之间的深小孔，深径比能达300:1以上，可 直接在工件的斜面、曲面进入加工。主要用于模具上千孔的 加工以及穿丝孔、化纤喷丝头、喷丝板的喷丝孔、滤板、滤

板筛板的群孔、发动机叶片、缸体的散热孔、液压、汽动阀 体的油路、汽路孔等，也可用于蚀除折断在工件中的钻头、 丝 锥 等 。

Scope of application:

The   electrode   tube    (brass   tube,coper    tube)is   used    as   the   tool  electrode.The    hihg    frequency    pulse     power    is    applied     betveen the     electrode      (hollow      copper     tube)and      the     workpiece.The  corrosion   rate   of   the    workpiece    is    greatly   accelerated,especial- ly         for         stainless         steel,hardened         steel,copper,aluminum,  cemented   carbide   and   other   conductive   materials   can   produce dep    holes    with    diameters    of    0.15mm-3.0mm,and    the    ratio    of depth   to   diameter   can    reach    up   to    more   than   300:1.It   can  directly     penetrated     from     the      inclined     surface     and      curved  surface   of   the   workpiece   It    is   mainly   used   for   machining   thou- sand   holes    on   the    mold,as   well    as   thread    hole,chemical    fiber spinneret   spinneret    hole   of   spinneret    filter    plate,group   holes    of sieve   plate    coolin   holes   of   engine    blades   cylinder    body,hydrau- lic   pressure,steam   valve    body   oil   way,steam    path   hole    and   so on.It   can   also   be   used   to    remove   the   bits   and   taps   that   are  broken   in   the   workpiece.

| XY 轴行程 XY axis travel |  |  | 350×450mm |
| --- | --- | --- | --- |
| 工作台尺寸 workbench size |  |  | 1040×550mm |
| 装夹电极直径 Diameter of clamp electrode |  | 0.15-3.0mm |  |
| 主轴高度 Spindle height |  | 1320mm |  |
| 打孔最大深度 |  | 300mm |  |
| 主轴头行程 |  | 380mm |  |
| 主轴伺服行程 Spindle servo stroke 主轴(Z轴伺服电机自动升降) Spindle(automatic lifting of Z-axis servo motor) XY轴丝杆 |  |  |  |
| 旋转头 Rotating head |  | 台湾原装 Taiwan original |  |
| 主轴电机 Spindle motor |  | 伺服电机 Servo motor |  |
| 编程输入 Programming input |  | 触摸屏CAD穿孔机编程一体化系统 |  |
| X,Y轴伺服驱动 |  | 伺服电机 |  |
| X,Yaxis servo drive |  | Servo motor |  |
| 加工速度 Processing speed |  | 60mm/min |  |
| 主轴最大调整速度 |  | 120mm/min |  |
| . 最大加工电流 Max.processing current |  | 35A |  |
| Rotating head/C axis rotation |  | 60-120rpm |  |
| 工作台面高度 Height of working table |  | 700mm |  |
| 气动泵 Pneumatic pump |  | 进口泵 |  |
| 输入最大功率 Input maximum power |  | 3.5kW |  |
| 工作液 |  | 纯净水 |  |
| Purfied water 工作液过滤方式 线隙式过滤 Working fluid fitration method Line gap filter |  |  |  |
| 夹具形式 Fixture form | 框型夹具 Frame type fixture |  |  |

★加工速度快 Fast processing speed

★自动靠边定位功能Automatic edge positioning function

★高精度光栅 High precision grating

★精密伺服马达Precise servo motor driving

★精密滚珠丝杆、直线导轨 Precise ball screw,linear guide rail

DD703-35

型号Type

500kg

三轴数控三轴数显

3-axis     CNC     3-axis     digital     display

工作台承重

Workbench    bearing X,Y,Z轴控制方式

X,Y,Z axis control mode

Max.depth of perforation

Spindle   head   stroke

350mm

台湾原装导轨

origina     guideway

Taiwan

双螺母精密丝杆

Double     nut     precison     screw

XY axis screw

The   integrated   control    system    of   the    rouch    screen   CAD    piercer

Max adjustment speed of spindle

旋转头/C轴旋转

Imput pump

Working fluid

XY位移精度                                                                         002mm

XY displacement accuracy

主机外型尺寸                                                                    1050×1050×2020mm

1m/min

最大位移速度

Max.displacement speed

Host size

★电火花穿孔机工作台尺寸、工作行程可定制。

13                                                                                                                                              14

HANYANG

DS703 Series Economical high-speed CNC spark-edm

经济型高速穿孔机的适用范围：

该穿孔机主要用于不锈钢，淬火钢，硬质合金、铜、铝等各 种导电材料工具上的深小孔加工。如电火花切割工件的穿丝 孔；化纤喷丝头、喷丝板上的喷丝孔；滤板、筛板上的群孔 ;发动机叶片、缸体的散热孔；液压、气动阀体的油路及气 路孔等。工作台X、Y 轴配有数显装置。使用纯净水或自来水 作为工作液。能直接从工件的斜面、曲面穿入加工，加工孔 的最大深径比可达200:7以上。

本机床也能用于蚀除折断在工件中的钻头、丝锥，而不损坏 原孔或螺纹。

Scope   of   application:

This  high  speed  CNC  Spark-EDM  is  mainly  used  for  deep  hole processing   on   tools   of  various   conductive   materials   such   as stainless     steel,hardened      steel,cemented      carbide,copper,and aluminum.Such  as  the  threading   holes  of  electric  spark   cutting workpieces;chemical    fiber     spinnerets,spinnerets     on     spinner- ets;group   holes  on  filter  plates  and  sieve   plates;heat  dissipa- tion    holes    for    engine    blades    and    cylinder    blocks;oil    for  hydraulic  and  pneumatic  valve  bodies   Road  and  air  passage holes,etc.The  XandY  axis  of  the  worktable  are  equipped  with digital  display  devices.Use  purified  water  or  tap  water  as  the working    fluid.It    can    directly     penetrated    from    the    inclined  surface   and   curved   surface   of   the   workpiece,and   the   maxi- mum  depth-to-diameter  ratio  of  the  processed  hole  can  reach up  to  more  than  200:1.

This  machine  tool  can  also  be  used  to  erode  drill  bits  and  taps broken  in  the  workpiece  without  damaging  the  original   hole  or thread.

★加工速度快 Fast processing speed

★自动靠边定位功能A utomatic edge positioning function

★高精度光栅 High precision grating

★精密伺服马达Precise servo motor driving

★精密滚珠丝杆、直线导轨 Precise ball screw,linear guide rail

主要技术参数    MAIN TECHNICAL DATA

| 型号 Type DS703 |  |
| --- | --- |
| XY 轴 行 程 XY axis travel | 300×400mm |
| workbench size 工作台尺寸 | 320×420mm |
| 工作台承重 Workbench bearing | 150kg |
| X,Y,Z轴控制方式 X.Y.Zaxis control mode | 1-axis CNC 2-axis digital display 单轴数控二轴数显 |
| 装夹电极直径 Diameter of clamp electrode | 0.3-3.0mm |
| 打孔最大深度 Max.depth of perforation | 200mm |
| 主轴头行程 Spindle head stroke | 250mm |
| 主轴伺服行程 Spindle servo stroke | 330mm |
| 主轴(Z轴伺服电机自动升降) Spindle(automatic lifting of Z-axis servo motor) | 台湾原装导轨 Taiwan original guideway |
| XY轴丝杆 XY axis screw | 滚珠丝杆 Ball screw rod |
| Rotating head 旋转头 | Taiwan origina 台湾原装 |
| 主轴电机 Spindle motor | 混合步进电机 Hybrid stepper motor |
| 编程输入 Programming input | 按钮式编程系统 Push button programming system |
| 加工速度 Processing speed | 30mm/min |
| 主轴最大调整速度 Max.adjustment speed of spindle | 120mm/min |
| 最大加工电流 Max.processing current | 30A |
| 旋转头/C 轴旋转 Rotating head/C axis rotation | 60-120rpm |
| 工作台面高度 Height of working table | 700mm |
| 气动泵 Pneumatic pump | 高压泵 High-pressure pump |
| Input maximum power 输入最大功率 | 3.5kW |
| 工作液 Working fluid | 纯净水 Purfied water |
| 工作液过滤方式 Working fluid filtration method | 线隙式过滤 Line gap filter |
| 夹具形式 Fixture form | 框型夹具 Frame type fixture |
| XY位移精度 XY displacement accuracy | 0.02mm |
| 主机外型尺寸 Host size | 1200×900×1980mm |

★电火花穿孔机工作台尺寸、工作行程可定制。

经济型高速电火花穿孔机

CNC MACHINE TOOL 汉洋数控

15                                                                                                                                              16

CNC 数控砂线切割机

SXH series CNC abrasive sand wire cutting machine

主要用于石墨、非金属、脆性材料等加工领域；实现异形切割，可实 现2D 图形的切割，与传统带锯条切割相比具备效率高、节约材料、耗 材成本低等明显优势设备稳定性和切割精度高。

推送料和接料装置方便了使用时的装夹和加工过程中的稳定，加工完 成后一次性取料的特性，可根据客户要求加装高压多点吸盘装置，应 对各种复杂材料。

可根据客户特殊要求和使用习惯定制各种规格的机型，可根据不同要 求的客户实现设备全防护、周边自动化等个性化需求。

Mainly used in the non-electrical-conductive material processing such as stone cutting,non-metallic,brittle  materials,etc.,it can achieve  irregular  cutting and can cut 2D graphics.Compared with traditional band saw blade cutting,it has obvious advantages such as high efficiency,material saving,and low consumable costs.The equipment has high stability and cutting accuracy.

The feeding and accepting devices facilitate clamping stability during processing.With the characteristic of one-time  material  retrieval after processing,high-pressure   multi-point   suction   cup   devices   can   be  installed  according  to  customer  requirements  to  cope  with  various  complex materials.

We can customize various specifications of models according to custom- ers'special  requirements  and  usage  habits,and  meet  personalized needs such as full equipment protection and peripheral automation for customers with different requirements.

HANYANG

CNC MACHINE TOOL

汉洋数控

| MAIN TECHNICAL DATA |  |  |  |
| --- | --- | --- | --- |
| 型号 MODEL | SXH450CNC | SXH630CNC | SXH800CNC |
| 最大回转直径 Max.Rotation Diameter | 450 mm | 630mm | 800mm |
| 最大加工高度 Max.processing height | 350 mm | 600mm | 800 mm |
| 最大加工长度 Max.processing length | 600 mm | 1400 mm | 1500 mm |
| 砂线轮槽数 Number of sand wheel grooves | 4 |  |  |
| 主轴转速范围 Spindle rotary speed range | 0-5000 rpm |  |  |
| 主电机功率 Main motor power | 2.2KW | 3KW | 3KW |
| 主轴电机类型 Mainly motor type | 变频/伺服VFD/servo |  |  |
| 导线轮直径 Sand Line Wheel Diameter | 220mm | 300mm | 350mm |
| X轴行程 X-axis travelling | 450 mm | 630 mm | 800 mm |
| Y轴行程 Y-axis travelling | 600 mm | 1400 mm | 1500 mm |
| XY轴驱动 XY axis driven | 伺服Servo |  |  |
| X向快移速度 X-direction fast movement speed | 6m/min |  |  |
| Y向快移速度 Y-direction fast movement speed | 6m/min |  |  |
| X向最小设定单位 The min.setting unit in Xdirection | 0.001 mm |  |  |
| Y向最小设定单位 The min.setting unit inY direction | 0.001mm |  |  |
| 重复 Repetitive positioning 定位精度 accuracy | X轴X-axis:0.012mm Y轴Y-axis:0.016mm |  |  |
| 精密气动张丝 Precision pneumatic wire tensioning | 气压控制Air pressure control |  |  |
| 气源 Gas source | 0.6Mp |  |  |
| 电源 Power supply | AC380V 50Hz |  |  |
| 整机功率 Whole machine power | 4KW | 7KW | 7.5KW |
| 伺服电机 Servo Motor 输出扭矩 Output Torque | X轴X-axis:7.16NM Y轴Y-axis:716NM | X轴X-axis:21.5NM Y轴Y-axis:28.6NM | X轴X-axis:21.5NM Y轴Y-axis:28.6NM |
| CNC系统 CNC system | 维宏NC60C Weihong NC6OC |  |  |
| 普通数控砂线系统 Ordinary CNC Sand Line System | XUL/HL |  |  |
| 切片系统 Cutting Control Program | 汉洋HANYANG |  |  |
| 整机重量 Machine Weight | 2300KG | 5200KG | 2800KG |
| 外形尺寸 Machine Size | 1800×1800×1450 | 3520×2000×2000 | 2400×1800×1850 |

17                                                                                                                                              18

环型平切砂线机

loop type horizontal abrasive-wire cutting machine

主要用于石墨、非金属、脆性材料等无材料加工领域；实现异形切割，可实现2D 图形的切割，具备效率高、节约材料 、耗材成本低等明显优势，设备稳定性和切割精度高。

升降稳定，加工完成后一次性取料，装夹方便，可根据客户要求加装附加工作台带旋转工作台，加工更快捷。

可根据客户特殊要求和使用习惯定制各种规格的机型，可根据不同要求的客户实现设备全防护、周边自动化等个性化需 求 。

Mainly  used  in  processing  such  as  graphite,non-metallic,

brittle  materials,etc;Realizing  irregular  cutting,it  can  achieve

cutting of 2D graphics,with obvious advantages such as high

equipment stability and cutting accuracy.

efficiency,material  savings,low  consumable  costs,and  high

Stable  lifting  and  lowering,one-time  material  retrieval  after

processing,convenient  clamping,and  can  be  equipped  with

additional    workbenches,such    as    rotating    workbenches

We can customize various specifications of models according

according to customer requirements for faster processing.

to   customers'special   requirements   and   usage   habits,and

meet  personalized  needs such  as full equipment  protection

and  peripheral  automation  according  to  different  require-

ments of customers.

19

HANYANG

CNC      MACHINE      TOOL

汉洋数控

| 主要技术参数 MAIN TECHNICAL DATA |  |
| --- | --- |
| 型号 Model | SXHP |
| 导线轮直径 Wire Wheel Diameter | φ220mm-350mm |
| 最大工件宽度 Max.Workpiece Width | 600mm-1000mm |
| 最大加工高度 Max.Processing Height | 100-1000mm |
| 旋转工作台 Rotating Worktable | 选配 Optional |
| 砂线轮槽数 Number of sand wheel grooves | 4 |
| 主轴电机类型 Spindle motor type | 变频电机 Variable frequency motor |
| 主轴转速范围 Spindle speed range | 0-5000 rpm |
| 主电机功率 Main motor power | 2.2KW |
| XZ轴驱动 XZ axis driven | 伺服 Servo |
| 重复定位精度 Repetitive positioning accuracy | X轴：0.012mm Z轴：0.016mm X-xis:0.012mm Z-axis:0.016mm |
| 控制系统 Control system | XUL和专用切片系统 XULand dedicated slicing system |

20

HANYANG

加工中心

VML Series CNC Machining Center

CNC MACHINE TOOL 汉洋数控

The  column  adopts  a   herringbone  structure

design to enhance the rigidity of the fuselage

and the anti twisting force during heavy cutting.

The machine tool itself comes standard with a

spindle    cooling    system,    ensuring    stable

machining  quality  during  high-speed  machin-

ing.

Standard configurations

★Ball screw -Taiwan  made  C3  grade

★Track-Taiwan    HIWIN,etc

★Taiwan cylinder

★Coupling-(R+W)

★Guideway  cover

★Volumetric  lubricating  oil  circuit

★Japanese    made    bearings-(NSK,NTN)

主要技术参数 MAIN TECHNICAL DATA

Model

Worktable size(mm)

Xaxis travelmm)

Y travel(mm)

Z travel(mm)

Distance from spindle center to z axis guide surface(mm)

Distance between spindle nose end to worktable(mm)

Table T-slot

(Number -Sizex spacing)

Spindle taper

Spindle speed

Ball screw spec

Positional accuracy

Repeated positioning accuracy

Spindle motor

Maximum load of table (kg)

Feed speed

Tool magazine form

(manipulator or bucket vertical)

Number of tools

Machine weight(kg)

System (optional)

Notes

manipulator or bucket vertical,optional

24          24          24          24

5900        6400        6500        6800

Mitsubishi,Siemens,Fanuc are optional

machine models,the specific specifications are subject

900*2000

1800

900

1000

950

200-1200

5-18*147.5

810*1700

1500

800

800

865

85-885

5-18*147.5

710*1360

1200

700

700

765

75-775

5-18*152.5

810*1500

1300

800

800

865

85-885

5-18*147.5

400*720

600

400

420

560

110-610

3-18*144

650*1200

1100

650

750

700

95-845

5-18*120

550*1000

800

550

550

560

140-690

5-18*90

500*1000

800

500

600

560

110-710

5-18*90

5010

±0.006

±0.004

15-18.5

2000

0-36000

4012

±0.005

±0.003

7.5-11

500

0-36000

5010

±0.006

±0.004

15-18.5

1500

0-36000

5010

±0.005

±0.004

15-18.5

1200

0-36000

4012

±0.004

±0.003

11-15

800

0-36000

4012

±0.005

±0.004

11-15

1000

0-36000

4012

±0.005

±0.003

5.5-7.5

400

0-36000

4012

±0.005

±0.003

7.5-11

500

0-36000

to the actual conditions.

24

13000

24

11000

24

5500

24

5800

there are many

8000/10000

8000/10000

8000/10000

8000/10000

VML1890

VML1580

VML1165

VML1270

VML1380

Because

VML640

VML850

VML855

8000

8000

8000

8000

BT50

BT50

BT50

BT40

BT40

BT50

BT40

BT40

22

生产检测设备

Production   testing   equipment

客户案例

Customer  case

中国兵器集团

China  North  Industries  Group

中国航空工业集团

Aviation Industry Corporation of China,LTD.

比亚迪股份有限公司

BYD Co., Ltd.

北京三一重机有限公司

YD

Beijing Sany Heavy Machinery Co.,Ltd.

SANY

中国南车

China South Locomotive

中国南车

上海金伟片板膜设备制造有限公司

CSR

# JWELL.

Shanghai Jinwei Film Equipment Manufacturing Co.,Ltd.

长沙中南大学

Changsha  Central   South  University

TO             O

东野精机(昆山有限公司

Dongye   Precision   Machinery   (Kunshan)Co.,Ltd.

湖南矿冶研究所

Hunan  Institute  of  Mining  and  Metallurgy

湖南有色新材料有限公司

Hunan Nonferrous New Materials Co.,Ltd.

ZOECO

中欧国际集团

湖南湘电长沙水泵有限公司

Hunan Xiangdian Changsha Water Pump Co.,Ltd.

上海中欧汽车电器有限公司

潮电长泵 B

Shanghai  Zhongou  Auto  Electric  Co.,Ltd.

沃得农机

江苏沃得农业机械配件制造有限公司

Jiangsu Wode Agricultural Machinery Parts Manufacturing Co.,Ltd.

哈电集团有限公司

哈电集团

Harbin Electric Group Co.,Ltd.

柳州上汽通用五菱集团

Liuzhou  SAIC-GM-Wuling  Group

哈尔滨东安发动机(集团)制造有限公司

上2用五

Harbn   Dongan    Engine   (Group)Manufacturing    Co.,Ltd.

DAE

柳州长虹机汽有限公司

Liuzhou Changhong Machinery &Steam Co.,Ltd.

JD长虹

玉柴

沈阳飞机研究所粉体有限公司

Shenyang Aircraft Research Institute Powder Co.,Ltd.

广西玉柴集团

Guangxi Yuchai Group

湖南湘潭电机股份有限公司 Hunan Xiangtan Electric Co.,Ltd.

谢电取份

广西科技大学

Guangxi University of Science and Technology

湖南国防科技大学

⑥ 度西科技大學

Hunan National University of Defense Technology

广西民族大学

Guangxi University for Nationalities

长沙黑金刚实业有限公司

黑金刚

Changsha  Black  King  KongIndustrial  Co.,Ltd.

广西柳州华南船舶机械有限公司

Guangxi Liuzhou South China Ship Machinery Co.,Ltd.

湖南天雁机械有限责任公司

TYEN天雁

Hunan  Tianyan  Machinery  Co.,Ltd.

销售网络

Sales network

俄罗斯Russia

英 国Britain

法国  France

西班牙 Spain

尼日利亚 Nigeria

波兰 C Poland

安哥拉 Angola

| 哈萨克斯坦 O Kazakhstan 印度 India | 缅甸 Myanmaoo. 泰 国O Thailand | 日 本Japan 韩 国 Korea 中 国 台 湾 越南 China.Taiwan Vietha m o 菲律宾Philipines 柬埔寨 Cambodia | 加拿大 Canada 美国 America |
| --- | --- | --- | --- |

马来西亚 Malaysia

巴西 Brazil

澳大利亚

乌拉圭 Uruguay

Australia

阿根廷

Argentina

始终坚持以客户的满意作为工作的中心， 保持技术的领先，服务的领先，

以确保市场的领先。

我们建立了遍及全国的销售网点，

…

……

我们不断地取得成功 … 我们不断再努力 ……

我们的销售将不断延伸

Insists always with the cente of the customer's satisfaction conduct and actions work, the keeping is in the lead technically,serving of be in the lead,to insure the market.

we established the all over global sale net orders, We obtain the success constantly … …

We work hard again continuously … …

Our sale won't break the extension..